STUDY DESCRIPTION LU-1

Project Roads and Trails

Condition Assessment

Draft – 4/20/2026

This Study Description is developed to inform information gaps identified by Calaveras County Water District (CCWD) for the relicensing of its North Fork Stanislaus River Hydroelectric Project (Project).

STUDY AREA

The Study Area includes Project Roads and Trails used to access Project facilities to conduct Operation and Maintenance (O&M) activities. For the purposes of this Study, a “Project Road or Trail” is a road or trail segment that is used exclusively for the purposes of the Project, including access to Project recreation facilities, and that is not otherwise used for non-Project purposes. Due to this Project need, a Project Road or Trail segment is maintained by and at the sole expense of CCWD. Project Roads and Trails do not include “joint use” or “multi-purpose” road and trail segments (i.e., road and trail segments used by other parties to access non-Project facilities, such as electric lines, residences, or other areas). For the purposes of this Study, Project Roads or Trails do not include roads within Project recreation areas or Project recreation trails (i.e., trails whose sole purpose is for Project recreation). A list and description of Project Roads is provided in Attachment A. The Project does not include any non-recreation trails.

STUDY METHODS

Excluding Data Summary Development (i.e., Reporting), the study will be completed in one step: Data Collection at Project Roads, which is described below.

Step 1: Data Collection at Project Roads

CCWD will visit each Project Road listed in Attachment A to assess the road segments’ condition and identify obvious signs and potential risks of environmental damage. If a road is on federal lands managed by the United States Department of Agriculture, Forest Service, or United States Department of the Interior, Bureau of Land Management (BLM) and the federal land management agency has established maintenance levels for the road segment on its managed lands, CCWD will become familiar with those maintenance level requirements prior to the field assessment. Assessments will be made in comparison to those Forest Service or BLM maintenance levels. Forest Service Maintenance Levels are provided in Attachment B. Items to be assessed or confirmed in the field condition assessment will include:

- List, map, and confirm lengths (i.e., hundredths of a mile) and widths (feet); Forest Service or BLM maintenance level, if applicable; traffic classification; seasons of use; confirm travel way surface material (i.e., native, gravel/aggregate, paved, or combination); and CCWD’s use (i.e., type and frequency of CCWD vehicle use) of each Project Road. The confirmation will not entail any land survey work but will utilize field Global Positioning System (GPS) equipment and best estimate by field staff during the filed condition assessment.

- Assess the condition of the travel way surface treatment and estimate remaining useful life of travel way surface. Determine pavement conditions, if appropriate, using the industry standard Pavement Management System (PMS). If the road is on Forest Service or BLM managed lands, determine if the travel way surface treatment is consistent with the designated maintenance level of the road (see Attachments A and B).

- Assess the condition and locate culverts and other drainage structures, including noting any significant erosion (take photographs and note location of the culverts and drainage structures by GPS).

- List, describe, and map bridges or other water crossings (take photographs and note location by GPS). Identify widths (feet) for all bridges and posted weight limits for any bridges that were constructed by or on behalf of CCWD.

- Assess the condition and confirm the location of CCWD-maintained traffic control devices; safety features; directional, warning and information signage including signs along roads (photograph each and note location of the culverts and drainage structures by GPS), and document compliance of those CCWD-maintained features with current Manual on Uniform Traffic Control Devices (MUTCD) standards or Forest Service or BLM standards, if the sign occurs in federal lands.

- Assess the condition of CCWD-maintained gates or other traffic exclusion structures (e.g., boulders) structures (photograph each and note location of gates and traffic exclusion structures by GPS). Note whether gate is locked at all times or opened during periods, and what parties have locks on the gate.

- Identify, describe, and map sub-standard, unsafe, or hazardous traffic conditions such as site clearing (e.g., blind spots and poor sight distances) and inadequate clear zones, using the appropriate American Association of State Highway and Transportation Officials (AASHTO) standards for traffic levels (photograph each and note location by GPS).

- Identify, describe, and map the location of obvious slope instability along the travel way or that may affect the road segment (photograph each and note location by GPS). This will not include any geotechnical or ground disturbing activities but best estimate by field staff during the condition assessment.

- Estimate CCWD’s typical use and routine O&M activities of the Project Road segment (i.e., this is an estimate of CCWD’s use, not a traffic count survey).

- Identify, describe, and map location and size of specifically designated vehicle turnouts (photograph each and note location by GPS). A turnout for the purpose of the Study is an area cleared along the side of the road that is at least 20 feet long and allows cars to pass a slower moving vehicle, and at which a motorist can safely (e.g., clear lines of sight and adequate width) turn around. Vehicular turnouts do not include parking areas near the Project facility being accessed.

REPORTING

CCWD will compile the study data into a summary report, including maps. If the information is collected in a manner inconsistent with this plan, the report will include a description of any inconsistency and why it occurred. Study data will be made available to all interested parties (See Concepts and Practices Applicable to All Relicensing Studies) and standard GIS shapefiles, including metadata, will be provided to the Forest Service.

SCHEDULE

CCWD anticipates the schedule for Study completion as follows:

Data Collection at Project Roads		April – September 2027

Reporting					October – December 2027

REFERENCES

USFS. June 2012. Guidelines for Road Maintenance Levels. 7700-Transportation Management 1177 1811-SDTDC.

USFS. March 2022. Forest Service Manual 7700 – Travel Management. Chapter 7730 – Transportation System Operation and Maintenance.

ATTACHMENT A

List of Project Roads in Study LU-1.

| Road Segment Designation |  | Termini (Lat/Long) |  | Est. Length (mile) | Est. Width (~feet) | Est. Driving Surface | Gated | Land Ownership | Current Maintenance Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Forest Service Designation | Start | End |  |  |  |  |  |  |
| NFDD Access Rd | 7N17B | 38.44039733443908, -120.00852306396033 | 38.43805295682596, -120.01707424824865 | 0.57 | 14 | Gravel | Yes | NFS | Forest Service |
| NSM Water Tank Access Road | Unnamed Extension off 7N01 | 38.405547943620526, -120.00461105098194 | 38.40503465244793, -120.00421889622864 | 0.04 | 14 | Native | No | NFS | CCWD |
| MPDD Access Road | Unnamed Extension off 4N38 | 38.23305529769888, -120.29545276032123 | 38.23378357483741, -120.29311575448797 | 0.14 | 13 | Gravel | Yes | NFS | CCWD |
| McKays LLO Access Road | Unnamed Extension off 4N38 | 38.23290512909011, -120.29553293431842 | 38.23305914803599, -120.29331640745194 | 0.13 | 13 | Gravel | Yes | NFS | CCWD |
| Beaver Creek Top of Penstock Access Road | 4N38D | 38.2358069, -120.2824934 | 38.2349360, -120.2837234 | 0.09 | 12 | Native | No | NFS | Forest Service |
| Beaver Creek Top of Penstock Access Road | Unnamed Extension off 4N38D | 38.23968934855315, -120.28164817438875 | 38.2358069, -120.2824934 | 0.23 | 12 | Native | No | Private | SPI |
| Beaver Creek Comms Tower Access Road | 4N38E | 38.23468513821321, -120.281137100000 | 38.231183317281584, -120.28211351755967 | 0.26 | 12 | Native | Yes | NFS | CCWD |
| Beaver Creek Access Road | 4N38D | 38.235801700309494, -120.27895108335633 | 38.23418864617101, -120.27933286675513 | 0.16 | 16 | Gravel | Yes | NFS | CCWD |
| Mill Creek Tap Access Road | Not Applicable | 38.191596860630966, -120.35392393679835 | 38.192477359900515, -120.35497497494651 | 0.14 | 12 | Paved | Yes | Private | CCWD / UWPA |
| Adit 4 Access Road | Not Applicable | 38.15741912992568, -120.38959526847506 | 38.154620110630475, -120.39179981973696 | 0.23 | 18 | Gravel | Yes | BLM & Private | CCWD |
| 10 Road Segments |  | -- |  | 1.99 | -- |  |  |  |  |

Key:

~ = Approximate								NFDD = North Fork Diversion Dam

BLM = United States Department of the Interior, Bureau of Land Management		NFS = National Forest System lands

CCWD = Calaveras County Water District					NSM = New Spicer Meadows Reservoir

Comms = Communications							LLO = Low Level Outlet

MPDD = McKays Point Diversion Dam

ATTACHMENT B

Table B-1. Summary of Forest Service Maintenance Levels.1

| Parameters | Maintenance Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| Service Life | Intermittent Service-Closed Status | Constant Service or Intermittent Service-Open Status (Some uses may be restricted under 36 Code of Federal Regulations Section 261.50) |  |  |  |
| Traffic Type | Open for non-motorized uses; Closed to motorized traffic. | Administrative, permitted, dispersed recreation specialized, commercial haul | All National Forest Traffic – General Use, Commercial Haul |  |  |
| Vehicle Type | Closed - N/A | High clearance, pick-up, 4x4, log trucks, etc. | All types - passenger cars to large commercial vehicles |  |  |
| Traffic Volume | Closed - N/A | Traffic volume increases with maintenance level |  |  |  |
| Typical Surface | All types | None; Native, or Aggregate – may be dust abated |  | Aggregate – usually dust abated; paved |  |
| Travel Speed | Closed - N/A | Travel speed increases with maintenance level |  |  |  |
| User Comfort and Convenience | Closed - N/A | Not a consideration | Low priority | Moderate priority | High priority |
| Functional Classification | All types | Local collector | Local collector arterial | Local collector arterial | Local collector arterial |
| Level of Service | Closed - N/A | J | G, H, I - Traffic service level increases with maintenance level |  |  |
| Management Strategy | Prohibit or eliminate | Discourage or prohibit cars. Accept or discourage high clearance vehicles | Encourage, accept | Encourage | Encourage |

1 	See also USDA Forest Service. 2012. Guidelines for Road Maintenance Levels. National Technology and Development Program. 7700 Transportation Management 11771811-SDTDC. June 2012., Forest Service Handbook. 7709.59, secs. 11 and 62.3., and Manual on Uniform Traffic Control Devices. Available online at: http://mutcd.fhwa.dot.gov/.

Key:
J = Traffic flow is slow and may be blocked by management activities. Two-way traffic is difficult, backing may be     
      required. Rough and irregular surface. Travel with low clearance vehicles is difficult. Single purpose facility.
G = Free flowing, mixed traffic; stable, smooth surface. Provides safe service to all traffic.
H = Congested during heavy traffic, slower speeds, and periodic dust; accommodates any legal-size load or vehicle.
I = Interrupted traffic flow, limited passing facilities, may not accommodate some vehicles. Low design speeds. 
     Unstable surface under certain traffic or weather.
N/A = Not applicable

In addition to the details provided in this study description, study implementation will also follow the "Concepts and Practices Applicable to All Relicensing Studies" developed by CCWD.

The condition of roads and trails within Project recreation areas will be included in the Pre-Application Document (PAD).

In addition to the details provided in this section, reporting will also follow the "Concepts and Practices

Applicable to All Relicensing Studies" developed by CCWD.